## MPC6535板上接线端子定义/MPC6535 Interfaces

### 引脚定义

| 分组 | 引脚定义 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Pow | 24V电源正 | 24V电源地 |  |  |  |  |  |
| X4 | 脚踏开关 | 开盖保护 | 水保护 | 24V地 | 24V正 |  |  |
| X1 | Z轴正限位 | Z轴负限位 | Z轴原点 | 24V地 | 24V正 |  |  |
| X2 | Y轴正限位 | Y轴负限位 | Y轴原点 | 24V地 | 24V正 |  |  |
| X3 | X轴正限位 | X轴负限位 | X轴原点 | 24V地 | 24V正 |  |  |
| Y2 | Z轴脉冲 | Z轴方向 | 5V正 |  |  |  |  |
| Y3 | Y轴脉冲 | Y轴方向 | 5V正 |  |  |  |  |
| Y4 | X轴脉冲 | X轴方向 | 5V正 |  |  |  |  |
| Y5 | 模拟量1 | 模拟量2 | PWM+ | PWM- | 激光开关+ | 激光开光- | 激光电源地 |
| Y1 | 吹气 | 加工结束 | U盘指示 | 保留 | 24V地 |  |  |

POWER：输入电源接口（24VDC）

说明：MPC6535采用单24VDC供电，其他电源引脚为控制器提供的输出电源。

### PIN ARRAY

| 分组 | Pin Array |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Pow | 24V | 24V GND |  |  |  |  |  |
| X4 | Foot Switch | Uncapping protection | Water protection | 24V GND | 24V |  |  |
| X1 | Z-axis Forward Limit | Z-axis Reverse Limit | Z-axis Origin | 24V GND | 24V |  |  |
| X2 | Y-axis Forward Limit | Y-axis Reverse Limit | Y-axis Origin | 24V GND | 24V |  |  |
| X3 | X-axis Forward Limit | X-axis Reverse Limit | X-axis Origin | 24V GND | 24V |  |  |
| Y2 | Z-axis Pulse | Z-axis Direction | 5V |  |  |  |  |
| Y3 | Y-axis Pulse | Y-axis Direction | 5V |  |  |  |  |
| Y4 | X-axis Pulse | X-axis Direction | 5V |  |  |  |  |
| Y5 | DA1 | DA2 | PWM+ | PWM- | Laser On/Off + | Laser On/Off - | Laser Power GND |
| Y1 | Blow-off | Processing Finished | USB Flash Disk Indication | Reserve | 24V GND |  |  |

X1: Input power interface (24VDC)

Note: MPC6535 adopts single 24VDC power supply. The other power pins are output.